Characteristics Study of Side Band Power And Frequencies in AM Using MATLAB

Aim:

To generate an Amplitude Modulated (AM) signal in MATLAB using specified carrier and modulating signal parameters

Program Code:

clc;

clear;

close all;

% Parameters

Am = 1;          % Message amplitude

Ac = 2;          % Carrier amplitude

fm = 1000;       % Message frequency (Hz)

fc = 10000;      % Carrier frequency (Hz)

Fs = 100000;     % Sampling frequency

t = 0:1/Fs:0.01;

mu = Am/Ac;

m = Am*cos(2*pi*fm*t);

c = Ac*cos(2*pi*fc*t);

s = Ac*(1 + mu*cos(2*pi*fm*t)).*cos(2*pi*fc*t);

figure;

subplot(3,1,1);

plot(t,m);

title('Message Signal');

xlabel('Time (s)');

ylabel('Amplitude');

subplot(3,1,2);

plot(t,c);

title('Carrier Signal');

xlabel('Time (s)');

ylabel('Amplitude');

% Plot AM signal

subplot(3,1,3);

plot(t,s);

title('AM Signal');

xlabel('Time (s)');

ylabel('Amplitude');

USB = fc + fm;

LSB = fc - fm;

fprintf('Carrier Frequency = %d Hz\n',fc);

fprintf('Lower Sideband Frequency = %d Hz\n',LSB);

fprintf('Upper Sideband Frequency = %d Hz\n',USB);

Pc = Ac^2/2;

Psb_total = Pc*(mu^2/2);

Pt = Pc + Psb_total;

fprintf('Carrier Power = %.2f W\n',Pc);

fprintf('Total Sideband Power = %.2f W\n',Psb_total);

fprintf('Total AM Power = %.2f W\n',Pt);

OUTPUT: