MATLAB-BASED MODELLING OF VSB MODULATOR AND DEMODULATOR FOR ANOLOG TELEVISION COMMUNICATION APPLICATIONS

AIM:

i) To generate and analyse the Vestigial Sideband (VSB) modulated signal by performing DSB-SC modulation and applying a suitable filter using MATLAB.

Ii)  To implement coherent modulation and recover the original message signal from the VSB signal, and compare the transmitted and recoverd waveforms.

PROGRAM CODE:

clc;

clear;

close all;

% Parameters

Am = 1;

Ac = 2;

fm = 1000;

fc = 10000;

Fs = 100000;

t = 0:1/Fs:0.01;

% Modulation index

mu = Am/Ac;

% Message signal

m = Am*cos(2*pi*fm*t);

% Carrier

c = Ac*cos(2*pi*fc*t);

% AM signal

s = Ac*(1 + mu*cos(2*pi*fm*t)).*cos(2*pi*fc*t);

% VSB filter

% Design a bandpass filter around carrier

f1 = 8500;

f2 = 11500;

[b,a] = butter(4,[f1 f2]/(Fs/2),'bandpass');

% VSB signal

vsb = filter(b,a,s);

% Demodulation

demod = vsb .* cos(2*pi*fc*t);

% Low pass filter

fcut = 2000;

[b2,a2] = butter(4,fcut/(Fs/2),'low');

output = filter(b2,a2,demod);

% Plot signals

figure;

subplot(4,1,1);

plot(t,m);

title('Message Signal');

xlabel('Time');

ylabel('Amplitude');

subplot(4,1,2);

plot(t,s);

title('AM Signal');

xlabel('Time');

ylabel('Amplitude');

subplot(4,1,3);

plot(t,vsb);

title('VSB Signal');

xlabel('Time');

ylabel('Amplitude');

subplot(4,1,4);

plot(t,output);

title('Demodulated Output');

xlabel('Time');

ylabel('Amplitude');

OUTPUT: