MATLAB SIMULATION OF SUPERHETERODYNERECEIVER FOR AM COMMUNICATION SYSTEM WITH FREQUENCY MIXING AND FILTERING

AIM:
	i) To model and simulate the operation of a superheterodyne receiver for an AM communication system using MATLAB by implementing frequency mixing and intermediate frequency (IF)

ii) To recover the original message signal through intermediate frequency filtering and envelope detection, and analyze the waveforms of the message, carrier, AM, IF, and recovered signals.

PROGRAM CODE:

clc;

clear;

close all;

% Parameters

Fs = 1e6;

t = 0:1/Fs:0.005;

fm = 5000;       % Message frequency

fc = 100000;     % RF carrier frequency

flo = 145000;    % Local oscillator frequency

Am = 1;

Ac = 2;

% Modulation index

mu = Am/Ac;

% Message signal

m = Am*cos(2*pi*fm*t);

% RF carrier

carrier = Ac*cos(2*pi*fc*t);

% AM signal

rf = Ac*(1 + mu*cos(2*pi*fm*t))...

.*cos(2*pi*fc*t);

% Local oscillator

lo = cos(2*pi*flo*t);

% Mixer

mixed = rf .* lo;

% IF frequency

fif = abs(flo-fc);

fprintf('RF Frequency = %.0f Hz\n',fc);

fprintf('Local Oscillator Frequency = %.0f Hz\n',flo);

fprintf('IF Frequency = %.0f Hz\n',fif);

% IF Bandpass Filter

f_low = 40000;

f_high = 50000;

[b,a] = butter(4,[f_low f_high]/(Fs/2),'bandpass');

if_signal = filter(b,a,mixed);

% Envelope detection

envelope = abs(hilbert(if_signal));

% Remove DC component

[b2,a2] = butter(4,10000/(Fs/2),'low');

output = filter(b2,a2,envelope);

% Plot

figure;

subplot(5,1,1);

plot(t,m);

title('Message Signal');

xlabel('Time');

ylabel('Amplitude');

subplot(5,1,2);

plot(t,rf);

title('Received AM Signal');

xlabel('Time');

ylabel('Amplitude');

subplot(5,1,3);

plot(t,mixed);

title('Mixer Output');

xlabel('Time');

ylabel('Amplitude');

subplot(5,1,4);

plot(t,if_signal);

title('Intermediate Frequency (IF) Signal');

xlabel('Time');

ylabel('Amplitude');

subplot(5,1,5);

plot(t,output);

title('Recovered Message');

xlabel('Time');

ylabel('Amplitude');

OUTPUT: