MATLAB SIMULATION OF SUPERHETERODYNE RECEIVER FOR AM COMMUNICATION SYSTEM WITH FREQUENCY MIXING AND FILTERING

Aim :

i) To model and simulate the operation of a superheterodyne receiver for an AM communication system using MATLAB by implementing frequency mixing and intermediate frequency (IF) signal generation.

ii) To recover the original message signal through intermediate frequency filtering and envelope detection, and analyze the waveforms of the message, carrier, AM, IF, and recovered signals.

Program Code:

clc;

clear;

close all;

%% Parameters

Fs = 100000;          % Sampling frequency

t = 0:1/Fs:0.02;      % Time

fm = 1000;            % Message frequency = 1 kHz

fc = 10000;           % AM carrier frequency = 10 kHz

fLO = 15000;           % Local oscillator = 15 kHz

fIF = 5000;            % Intermediate frequency = 5 kHz

Am = 1;                % Message amplitude

Ac = 2;                % Carrier amplitude

mu = 0.7;              % Modulation index

%% 1. Message Signal

m = Am*cos(2*pi*fm*t);

%% 2. AM Transmitted Signal

AM = Ac*(1 + mu*m).*cos(2*pi*fc*t);

%% 3. Local Oscillator Signal

LO = cos(2*pi*fLO*t);

%% 4. Frequency Mixing

% Mixer multiplies received AM signal with local oscillator

mixed = AM .* LO;

%% 5. IF Bandpass Filter

% Difference frequency = |fLO - fc|

% = |15 kHz - 10 kHz| = 5 kHz

bpFilt = designfilt('bandpassiir', ...

'FilterOrder', 6, ...

'HalfPowerFrequency1', 4500, ...

'HalfPowerFrequency2', 5500, ...

'SampleRate', Fs);

IF_signal = filtfilt(bpFilt, mixed);

%% 6. Envelope Detection

envelope = abs(hilbert(IF_signal));

%% 7. Low Pass Filter

lpFilt = designfilt('lowpassiir', ...

'FilterOrder', 6, ...

'HalfPowerFrequency', 1500, ...

'SampleRate', Fs);

demodulated = filtfilt(lpFilt, envelope);

%% 8. Plot Results

figure;

subplot(5,1,1);

plot(t, m);

xlabel('Time (s)');

ylabel('Amplitude');

title('Message Signal');

grid on;

subplot(5,1,2);

plot(t, AM);

xlabel('Time (s)');

ylabel('Amplitude');

title('AM Transmitted Signal');

grid on;

subplot(5,1,3);

plot(t, mixed);

xlabel('Time (s)');

ylabel('Amplitude');

title('Mixer Output');

grid on;

subplot(5,1,4);

plot(t, IF_signal);

xlabel('Time (s)');

ylabel('Amplitude');

title('Filtered IF Signal (5 kHz)');

grid on;

subplot(5,1,5);

plot(t, demodulated);

xlabel('Time (s)');

ylabel('Amplitude');

title('Recovered Message Signal');

grid on;

OUTPUT: