PERFORMANCE EVALUATION OF AM GENERATION AND DETECTION UNDER NOISY CHANNEL CONDITIONS USING MATLAB

Aim:

- To generate an Amplitude Modulated (AM) signal, introduce Additive White Gaussian Noise (AWGN), and analyze the effect of noise on the transmitted signal using MATLAB.

- ii) To recover the original message signal from the noisy AM signal using envelope detection and compare the transmitted and recovered waveforms to evaluate the performance of the AM communication system.

Program Code:

clc;

clear;

close all;

%% Parameters

Fs = 100000;

t = 0:1/Fs:0.02;

fm = 1000;

fc = 10000;

Am = 1;

Ac = 2;

mu = 0.7;

SNR_dB = 10;

%% 1. Message Signal

m = Am*cos(2*pi*fm*t);

%% 2. AM Signal Generation

AM_signal = Ac*(1 + mu*m).*cos(2*pi*fc*t);

%% 3. Add AWGN Noise Manually

signal_power = mean(AM_signal.^2);

SNR_linear = 10^(SNR_dB/10);

noise_power = signal_power/SNR_linear;

noise = sqrt(noise_power)*randn(size(AM_signal));

noisy_signal = AM_signal + noise;

%% 4. AM Detection

analytic_signal = hilbert(noisy_signal);

envelope = abs(analytic_signal);

%% 5. Recover Message

recovered = (envelope - Ac)/(Ac*mu);

%% 6. Low Pass Filter

lpFilt = designfilt('lowpassiir', ...

'FilterOrder', 6, ...

'HalfPowerFrequency', 1500, ...

'SampleRate', Fs);

recovered = filtfilt(lpFilt,recovered);

%% 7. Calculate MSE

MSE = mean((m - recovered).^2);

%% 8. Calculate Output SNR

signal_power_message = mean(m.^2);

error_power = mean((m - recovered).^2);

output_SNR = 10*log10(signal_power_message/error_power);

%% 9. Display Results

fprintf('---------------------------------------\n');

fprintf('AM PERFORMANCE UNDER NOISY CHANNEL\n');

fprintf('---------------------------------------\n');

fprintf('Message Frequency = %.2f Hz\n',fm);

fprintf('Carrier Frequency = %.2f Hz\n',fc);

fprintf('Modulation Index = %.2f\n',mu);

fprintf('Input SNR = %.2f dB\n',SNR_dB);

fprintf('Output SNR = %.2f dB\n',output_SNR);

fprintf('Mean Square Error = %.6f\n',MSE);

%% 10. Plot Signals

figure;

subplot(4,1,1);

plot(t,m);

xlabel('Time (s)');

ylabel('Amplitude');

title('Original Message Signal');

grid on;

subplot(4,1,2);

plot(t,AM_signal);

xlabel('Time (s)');

ylabel('Amplitude');

title('Generated AM Signal');

grid on;

xlim([0 0.003]);

subplot(4,1,3);

plot(t,noisy_signal);

xlabel('Time (s)');

ylabel('Amplitude');

title('AM Signal with Noise');

grid on;

xlim([0 0.003]);

subplot(4,1,4);

plot(t,recovered);

xlabel('Time (s)');

ylabel('Amplitude');

title('Recovered Message Signal');

grid on;

OUTPUT: