SIMULATION AND ANALYSIS OF UNIFORM AND NON UNIFORM QUANTIZATION USING MATLAB

Aim:

i)

To implement and analyze uniform and non-uniform quantization techniques for an analog signal using MATLAB  by converting the continuous amplitude signal into discrete quantization levels

ii)

To compare the performance of uniform quantization by analyzing their quantized waveforms and quantization errors, and evaluate their effectiveness in representing signals with different amplitude variations.

Program Code:

clc; clear; close all;

%% 1. Signal Generation

fs = 8000;                  % Sampling frequency (Hz)

t  = 0:1/fs:0.02;           % Time vector (20 ms)

f1 = 200;                   % Signal frequency (Hz)

Am = 1;                     % Peak amplitude

% Test signal: sine with varying amplitude (to show non-uniform advantage)

x = Am * sin(2*pi*f1*t) .* (0.2 + 0.8*abs(sin(2*pi*50*t)));

x = x / max(abs(x));        % Normalize to [-1, 1]

%% 2. Uniform Quantizer (Mid-Rise)

function xq = uniform_quant(x, nbits, xmax)

L     = 2^nbits;                    % Number of levels

delta = 2*xmax / L;                 % Step size

xq    = delta * (floor(x/delta) + 0.5);

% Clip to valid range

xq = min(max(xq, -xmax + delta/2), xmax - delta/2);

end

%% 3. Mu-law Compressor and Expander

function y = mu_compress(x, mu)

y = sign(x) .* log(1 + mu*abs(x)) / log(1 + mu);

end

function x = mu_expand(y, mu)

x = sign(y) .* ((1 + mu).^abs(y) - 1) / mu;

end

%% 4. Non-Uniform Quantizer (Companding)

function [xq, yq] = nonuniform_quant(x, nbits, mu)

y  = mu_compress(x, mu);            % Compress

yq = uniform_quant(y, nbits, 1);    % Uniform quantize compressed signal

xq = mu_expand(yq, mu);             % Expand

end

%% 5. Apply Quantization (n = 4 bits example)

nbits = 4;

mu    = 255;

xq_uni = uniform_quant(x, nbits, 1);

[xq_nu, yq_comp] = nonuniform_quant(x, nbits, mu);

% Quantization errors

err_uni = x - xq_uni;

err_nu  = x - xq_nu;

%% 6. Plot Time-Domain Results

figure('Name','Quantization Results','Position',[100 100 1000 700]);

subplot(3,2,1);

plot(t*1000, x, 'b', 'LineWidth', 1.2); grid on;

title('Original Signal'); xlabel('Time (ms)'); ylabel('Amplitude');

subplot(3,2,3);

plot(t*1000, x, 'b--'); hold on;

stairs(t*1000, xq_uni, 'r', 'LineWidth', 1.2); grid on;

title(sprintf('Uniform Quantization (%d-bit)', nbits));

xlabel('Time (ms)'); ylabel('Amplitude'); legend('Original','Quantized');

subplot(3,2,4);

plot(t*1000, err_uni, 'r'); grid on;

title('Uniform Quantization Error'); xlabel('Time (ms)'); ylabel('Error');

subplot(3,2,5);

plot(t*1000, x, 'b--'); hold on;

stairs(t*1000, xq_nu, 'g', 'LineWidth', 1.2); grid on;

title(sprintf('Non-Uniform (\\mu-law, \\mu=%d, %d-bit)', mu, nbits));

xlabel('Time (ms)'); ylabel('Amplitude'); legend('Original','Quantized');

subplot(3,2,6);

plot(t*1000, err_nu, 'g'); grid on;

title('Non-Uniform Quantization Error'); xlabel('Time (ms)'); ylabel('Error');

%% 7. Compander Characteristics

figure('Name','Compander Characteristics');

xin = linspace(-1, 1, 1000);

hold on;

for m = [1 50 100 255]

plot(xin, mu_compress(xin, m), 'LineWidth', 1.3, ...

'DisplayName', sprintf('\\mu = %d', m));

end

plot(xin, xin, 'k--', 'DisplayName', 'Linear (Uniform)');

grid on; legend('Location','southeast');

title('\mu-law Compression Characteristics');

xlabel('Input x'); ylabel('Compressed output y');

%% 8. Input-Output Transfer Characteristics

figure('Name','Quantizer Transfer Characteristics');

xin = linspace(-1, 1, 2000);

subplot(1,2,1);

stairs(xin, uniform_quant(xin, 3, 1), 'r', 'LineWidth', 1.3); hold on;

plot(xin, xin, 'k--'); grid on;

title('Uniform Quantizer (3-bit)'); xlabel('Input'); ylabel('Output');

subplot(1,2,2);

[xnu, ~] = nonuniform_quant(xin, 3, mu);

stairs(xin, xnu, 'g', 'LineWidth', 1.3); hold on;

plot(xin, xin, 'k--'); grid on;

title('Non-Uniform Quantizer (3-bit, \mu-law)');

xlabel('Input'); ylabel('Output');

%% 9. SQNR Analysis vs Number of Bits

bits_range = 2:10;

sqnr_uni = zeros(size(bits_range));

sqnr_nu  = zeros(size(bits_range));

for k = 1:length(bits_range)

nb = bits_range(k);

xu = uniform_quant(x, nb, 1);

[xn, ~] = nonuniform_quant(x, nb, mu);

sqnr_uni(k) = 10*log10(sum(x.^2) / sum((x - xu).^2));

sqnr_nu(k)  = 10*log10(sum(x.^2) / sum((x - xn).^2));

end

figure('Name','SQNR vs Bits');

plot(bits_range, sqnr_uni, 'r-o', 'LineWidth', 1.5); hold on;

plot(bits_range, sqnr_nu,  'g-s', 'LineWidth', 1.5);

plot(bits_range, 6.02*bits_range + 1.76, 'k--', 'LineWidth', 1);

grid on;

legend('Uniform','Non-Uniform (\mu-law)','Theoretical 6.02n+1.76 dB', ...

'Location','northwest');

title('SQNR vs Number of Bits');

xlabel('Number of bits (n)'); ylabel('SQNR (dB)');

%% 10. SQNR vs Input Amplitude (Dynamic Range Analysis)

amp_dB   = -50:2:0;                     % Input level in dB below full scale

sqnr_u_a = zeros(size(amp_dB));

sqnr_n_a = zeros(size(amp_dB));

xbase    = sin(2*pi*f1*t);

for k = 1:length(amp_dB)

a  = 10^(amp_dB(k)/20);

xs = a * xbase;

xu = uniform_quant(xs, 8, 1);

[xn, ~] = nonuniform_quant(xs, 8, mu);

sqnr_u_a(k) = 10*log10(sum(xs.^2) / sum((xs - xu).^2));

sqnr_n_a(k) = 10*log10(sum(xs.^2) / sum((xs - xn).^2));

end

figure('Name','SQNR vs Input Level (8-bit)');

plot(amp_dB, sqnr_u_a, 'r-o', 'LineWidth', 1.5); hold on;

plot(amp_dB, sqnr_n_a, 'g-s', 'LineWidth', 1.5);

grid on; legend('Uniform','Non-Uniform (\mu-law)','Location','south');

title('SQNR vs Input Signal Level (8-bit)');

xlabel('Input level (dB rel. full scale)'); ylabel('SQNR (dB)');

%% 11. Numerical Summary

fprintf('\n===== Quantization Analysis (n = %d bits) =====\n', nbits);

fprintf('Uniform     : MSE = %.6f | SQNR = %.2f dB\n', ...

mean(err_uni.^2), 10*log10(sum(x.^2)/sum(err_uni.^2)));

fprintf('Non-Uniform : MSE = %.6f | SQNR = %.2f dB\n', ...

mean(err_nu.^2),  10*log10(sum(x.^2)/sum(err_nu.^2)));

fprintf('Theoretical uniform SQNR (full-scale sine) = %.2f dB\n', ...

6.02*nbits + 1.76);