FM GENERATION AND DETECTION USING FM TRAINER KIT

Aim:

1. To generate and observe frequency modulated (FM) signals using the trainer kit.

2. Identify the variation of carrier frequency with the instantaneous amplitude of the message signal and to calculate the modulation index.

3. To detect the FM signal and analyze the recovered message signal.

4. Compare the recovered message waveform with the applied message signal and assess the quality of detection. Block Diagram for FM Modulator Block Diagram for FM Demodulator Model Graph

Program Code:

clc;

clear;

close all;

%% Parameters

Fs = 100000;              % Sampling frequency

t = 0:1/Fs:0.02;          % Time vector

fm = 1000;                % Message frequency = 1 kHz

fc = 10000;               % Carrier frequency = 10 kHz

kf = 5000;                % Frequency sensitivity

Am = 1;                   % Message amplitude

Ac = 1;                   % Carrier amplitude

%% 1. Message Signal

m = Am*cos(2*pi*fm*t);

%% 2. FM Generation

% FM signal:

% s(t) = Ac*cos(2*pi*fc*t + 2*pi*kf*integral(m(t)))

phase = 2*pi*fc*t + 2*pi*kf*cumsum(m)/Fs;

FM_signal = Ac*cos(phase);

%% 3. FM Detection

% Differentiate the FM signal

dFM = [0 diff(FM_signal)] * Fs;

% Obtain the envelope of differentiated signal

detected = abs(hilbert(dFM));

%% 4. Remove DC component

detected = detected - mean(detected);

%% 5. Low Pass Filter

lpFilt = designfilt('lowpassiir', ...

'FilterOrder', 6, ...

'HalfPowerFrequency', 1500, ...

'SampleRate', Fs);

recovered = filtfilt(lpFilt, detected);

%% 6. Normalize recovered signal

recovered = recovered / max(abs(recovered));

%% 7. Plot Results

figure;

subplot(4,1,1);

plot(t,m);

xlabel('Time (s)');

ylabel('Amplitude');

title('Message Signal');

grid on;

subplot(4,1,2);

plot(t,FM_signal);

xlabel('Time (s)');

ylabel('Amplitude');

title('Generated FM Signal');

grid on;

xlim([0 0.005]);

subplot(4,1,3);

plot(t,detected);

xlabel('Time (s)');

ylabel('Amplitude');

title('FM Detector Output');

grid on;

subplot(4,1,4);

plot(t,recovered);

xlabel('Time (s)');

ylabel('Amplitude');

title('Recovered Message Signal');

grid on;